Gcube-102 Linux Driver Install Guide.

Version : 1.0

Os : Ubuntu 16.04 64bit/32bit

Index

1. Serial Connection.

2. USB to Serial Connection .

3. USB Connection.

4. Ethernet Connection.

1. Serial Connection.

1) It must be run on terminal and root account.

- run terminal.

- type : ~# sudo -s

2) Printer set to serial interface mode.

3) Connect Serial cable with printer and power the printer on.

4) Make sure cable and printer is powerd on.

5) run ./cli_installer_gcube102_64bit  on terminal.

6) Select “1) Serial”.

7) The serial port on your pc will be detected automatically.

8) Select configuration of your serial port.

9) Done.

10) Open setting pannel.

11) Select “Printers” on setting pannel menu.

12) You can see your printer.

2. USB to Serial Connection.

1) It must be run on terminal and root account.

- run terminal.

- type : ~# sudo -s

2) Printer set to serial interface mode.

3) Connect USB-to-Serial cable with printer and power the printer on.

4) Make sure cable and printer is powerd on.

5) run ./cli_installer_gcube102_64bit  on terminal.

6) Select “2) USB to Serial”.

7) The serial port on your pc will be detected automatically.

8) Select configuration of your serial port.

9) Done.

10) Open setting pannel.

11) Select “Printers” on setting pannel menu.

12) You can see your printer.

3. USB Connection.

1) It must be run on terminal and root account.

- run terminal.

- type : ~# sudo -s

2) Printer set to USB interface mode.

3) Connect USB cable with printer and power the printer on.

4) Make sure cable and printer is powerd on.

5) run ./cli_installer_gcube102_64bit  on terminal.

6) Select “3) USB”.

7) The USB port on your pc will be detected automatically.

8) Done.

9) Open setting pannel.

10) Select “Printers” on setting pannel menu.

11) You can see your printer.

4. Ethernet Connection.

1) It must be run on terminal and root account.

- run terminal.

- type : ~# sudo -s

2) Printer set to Ethernet interface mode.

3) Connect LAN cable with printer and power the printer on.

4) You can check ip address to press [FEED]+Power on.

5) run ./cli_installer_gcube102_64bit  on terminal.

6) Select “4) Ethernet”.

7) Enter IP Address and Port Number, Port Number might be 6001.

8) Done.

9) Open setting pannel.

10) Select “Printer” on setting pannel menu.

11) You can see your printer.

Thank you.